Environmental requirements for an embedded motorcycle display

For a permanently integrated dashboard in tropical, high-UV, high-humidity conditions like the Philippines, IP65 is marginal. Aim higher on ingress protection, UV stability, temperature range, and vibration resistance.

Ingress protection and mechanical durability

- Target rating:

  - IP67 minimum (dust-tight; immersion to 1 m for 30 min).

  - IP68 if frequent heavy rain, pressure washing, or river crossings.

- Impact resistance:

  - Consider IK07–IK08 front-face impact protection to survive stones/debris.

- Sealing details:

  - Label: Use molded gasket (EPDM or silicone), compression-controlled with a groove.

  - Vent: Add a hydrophobic/oleophobic breather vent (ePTFE) to equalize pressure and reduce condensation.

  - Cable entry: Use rated cable glands and/or Deutsch DT/M12 connectors with backshells and strain relief.

- Materials:

- Front window: UV-stabilized polycarbonate or chemically strengthened glass with hard coat.

- Housing: Anodized aluminum or UV-stable ASA/PC-ABS; avoid standard ABS (UV embrittlement).

- Fasteners: Stainless steel (A2/A4) with anti-galvanic isolation.

UV, sunlight readability, and user interaction

- Display readability:

  - Label: 700–1000+ nits brightness; optical bonding to reduce reflection; anti-glare and anti-reflective coatings.

  - Polarizer: Choose orientation compatible with helmet visors/sunglasses.

  - Touch: Capacitive with glove mode or resistive if gloves are thick; add UV-stable cover lens.

- UV protection:

  - Use UV-resistant hard coat on the lens; specify materials rated for long-term tropical exposure.

- Condensation/fogging:

- Optical bonding + breather vent; optional anti-fog coating on the inner surface.

Thermal management and component ratings

- Operating temperature targets:

  - Design for -20 to +85°C ambient; sun-load on black housings can push internal temps higher.

- Electronics:

  - Select industrial/automotive-grade parts (e.g., MCU, PMICs, display drivers) rated to -40 to +85/105°C; derate power components.

  - Conformal coating (urethane or silicone) on PCBs to guard humidity and salt air; avoid acrylic in high-heat environments.

- Heat handling:

- Label: Thermal interface materials + aluminum housing as a heat spreader; avoid trapping heat near the display.

- Placement: Shield from engine/exhaust heat; provide small internal airflow paths without compromising sealing.

Electrical robustness and EMC

- Power conditioning:

  - Label: Wide-input DC-DC (9–36 V) with filtering; protect against load dump and transients (TVS diodes, input clamp, pi filters).

  - Support cranking voltages and brownouts; implement graceful boot and state recovery.

- EMC and noise:

  - Shielded cables, common-mode chokes on data lines, proper grounding strategy; consider ISO 7637-2 transients for vehicle supply.

- Interfaces (CAN/serial):

- Use automotive-grade transceivers; add ESD protection; route differential pairs tightly and keep away from HV ignition runs.

Vibration, mounting, and sealing of the assembly

- Vibration:

  - Design to ISO 16750-3 or MIL-STD-810 vibration profiles; use stiff PCB supports and elastomeric mounts.

- Mounting:

  - Isolate the display with bushings; avoid cantilevered masses; test on-road for resonance.

- Sealing longevity:

- Gaskets with defined compression stops; periodic serviceability without tearing seals.

Recommended baseline specification for this project

- Ingress: IP67 (prefer IP68 if pressure-washed)

- Impact: IK08 front lens

- Display: 5–7" TFT, 800+ nits, optically bonded, AR/AG coatings

- Touch: Capacitive with glove mode or resistive, UV-stable cover lens

- Materials: UV-stable polycarbonate lens; anodized aluminum or ASA housing

- Electronics: ESP32-S3 (industrial temp), automotive-grade CAN transceiver, conformal-coated PCB

- Power: 9–36 V input, surge/load-dump protection, TVS, LC filtering

- Connectors: Deutsch DT/M12, IP67 cable glands, strain relief

- Vent: ePTFE breather to mitigate condensation

Practical notes for the Philippines

- High humidity and salt air: Prioritize conformal coating, sealed connectors, and desiccant during shipment/storage.

- UV and heat: Light-colored housings reduce sun-load; avoid black where possible; ensure lens coatings are truly UV-stable.

- Rain intensity: Design for driving rain and occasional immersion; IP67/68 plus good cable management prevents wicking.